	Section Cover Page

	
Section 14 01 22

2008-06-02
Elevator Task and Frequency Schedule


This Master Specification Section contains:

.1
This Cover Sheet

.2
Specification Section Text:

1. Related Sections

2. Frequency and Tasks

Every Scheduled Visit

Monthly

Quarterly

Annually

Every Two Years

Every Five Years

1. Related Sections

.1
Elevator Device Maintenance:
Section 14 01 01

.2
Preventive Maintenance Log Sheet for Elevators
Section 14 01 23

.3
Hydraulic Elevator Oil Loss Monitoring Task and Frequency Schedule
Section 14 01 24

2.
Frequency and Tasks

.1
Every Scheduled Visit:

.1
Meet with Minister’s representative, identify and correct all concerns.

.2
Inspect condition of hoistways, pits and machine rooms, ensure they are clean and dry.

.3
Ride car, checking for unusual noise or operation.

.4
Complete all scheduled tasks within stated time frame.

.5
Perform all work required to maintain equipment in safe and reliable operating condition including but not limited to scheduled tasks.

.6
Complete the Minister’s 52-week Work Schedule System Log Sheets and record required information in Log Book.

.2
Monthly: 

.1
Inspect and maintain all parts of the machinery requiring lubrication.

.2
Check general operation, maintain and repair components as required.

.3
Test door reopening devices.

.4
Inspect and ensure indicating lights, lighting and gongs are operating properly

.5
Test telephone and emergency alarm bell and/or signaling device. 
.6
Inspect and operate governor.

.7
Complete oil loss monitoring requirements for all hydraulic elevators as per Section 14 01 24 

.3
Quarterly:

.1
Inspect and maintain all parts of the machinery and equipment requiring lubrication. 

.2
Inspect and maintain all landing and car door components to ensure safe and proper operation including but not limited to:
.1
interlocks, locks and contacts;

.2
door reopening devices;

.3
door operators, belts and linkages;

.4
vision panels;

.5
hoistway access switches;

.6
eccentrics/upthrusts and retainers;

.7
door gibs;

.8
pickup rollers and assemblies;

.9
clutch/retiring cams and assemblies;

.10
hanger/door panel interconnecting means;

.11
closer; and
.12
door panels and sight guards.

.3
Inspect, lubricate, clean, test and maintain all devices as required by code including but not limited to:

.1
car and counterweight safety mechanisms and governors;

.2
oil buffers and plungers;

.3
valve and cylinder packing glands

.4
oil tank level

.5
hydraulic cylinder

.6
controller contactors and relays;

.7
wire ropes and shackles;

.8
landing and car door closing force;

.9
restrictions on opening of car doors;

.10
driving machine brakes;

.11
car emergency lighting;

.12
free fall, overspeed and uncontrolled low speed protection devices;

.4
Inspect and ensure proper operation of emergency stop switch in car.

.5
Inspect and ensure the motor brushes are correctly seated and functioning properly.

.6
Inspect all resistance tubes and grids in controllers.

.7
Inspect and ensure proper operation of leveling switches.

.4
Annually:

.1
Clean chains, tracks and sheaves in bi-parting doors.  Lubricate tracks as necessary.  DO NOT lubricate chain.

.2
Dismantle and clean driving machine brakes and components.  Ensure proper operation.

.3
Test to ensure that the car decelerates from rated speed when power is removed from the driving machine and brake while an empty car is traveling in the up direction at rated speed.

.4
Check generator brushes.

.5
Lubricate car fan motor.

.6
For machines located over the hoistway:

.1
Hoisting ropes of elevators that have winding drums driving machines with 1:1 roping.

.1
Resocket babitted rope socket hoisting ropes.

.2
Replace fastenings or move them to a point above the existing fastening at the car ends.

.7
Perform a no-load leak-down test on all hydraulic elevators with buried hydraulic piping or buried hydraulic cylinders. Test duration shall be for a minimum of 30 minutes. Note the test results in the Log Book and provide a written detailed report to the minister within 30 days of the test.

.5
Every Two Years:

.1
For machines located below or at the side of the hoistway:

.1
Hoisting ropes of elevators that have winding drums driving machines with 1:1 roping.

.1
Resocket babitted rope socket hoisting ropes.

.2
Replace fastenings or move them to a point above the existing fastening at the car ends.

.6
Every Five Years:

.1
Perform a buffer return test as per code requirements on all car and counterweight buffers.

END OF SECTION

	PMSMS Property Management Services Master Specification
	

	Alberta Infrastructure
Master Specification System
	Page 0



