Amendment to Specification 5.11, Supply of Portland Cement

Subsection 5.11.2  Materials, shall be replaced in its entirety with the following:

The Contractor shall supply cement meeting the requirements of CSA A3000-18 Cementitious Materials Compendium, for the type of cement specified.

Unless otherwise approved by the Consultant, the specification requirements shall apply to the cement in the delivery vehicle at the cement storage site.

The Contractor shall, at his expense and to the satisfaction of the Consultant; prevent contamination of the cement, by cement of another type or by any other material; maintain records of times of receipt of cement delivery orders, cement departure from the source, arrival at the cement storage site, and start and completion of unloading, and provide this information to the Consultant upon request.

The Contractor shall provide the Consultant with a "Certificate of Compliance" acceptable to the Consultant, for the Portland Cement to be used in the Work.

The Consultant may verify the quality of the material at any time.  Sampling and testing for verification purposes shall meet the requirements of CSA A3000-18 Cementitious Materials Compendium.